V3688+ software upgrade program

This upgrade program is mainly for upgrade v3688+. Please refer to Hong Kong Service Bulletin or software release note for detail. Follows below are the procedures and notice of this upgrade program.

# Extract pcplus.exe file

- Flexing file uploading procedure

## The procedure of Flashing and Flexing the phone

- Flashing procedure only

- The important things before flashing and after flashing the phone

- Equipment and cables

# Please note that all the contents are strictly for your service purpose only. It should not be disclosed / released to any outside parties without the authorization from MAPAL.

# Extract pcplus.exe file

- Copy a:\pcplus.exe to c:\

- Backup or rename an original pcplus folder if your PC already installed it.

- Extract the pcplus.exe and press “y” to create a new folder for pcplus. (all files are inside this folder)

# Flexing file uploading procedure

- Enter pcplus directory and execute pcplus application by type “pcplus” then “enter”.

- Power on the EMMI box by press the blue button.

- Press any key to enter terminal mode.

- Type “flash” then press “enter” twice to connect Motorola FLASH Programmer.

- Screen should display:

### Motorola FLASH Programmer

- >

- At the > prompt type “ff” then press “enter” to uploading flex definition file.

- Press “Page up” and select ASCII protocol by press “a”.

- Enter directory and filename “c:\pcplus\v3688cke.fdf” then press “enter”.

- Upload is in progress.

- A “BEEP-BEEP” alert indicates that the file is uploaded.

- No need to upload this flexing file again otherwise the PC connection to EMMI box is disconnected, EMMI box is powered down or the pcplus program is quit.

## The procedure of Flexing the phone

- Connect v3688+ radio to the EMMI box through buttplug cable.

- Power on the phone and press function keys “F1, F2, F3, F4 and F5” to flex the phone by correct flex ID.

- If the message shows “flexing complete” indicated that the flexing process is completed.

- Power off the phone and detach the Buttplug cable.

- Power on the phone, enter test mode by press “#” key for 3 seconds then press “57#” to reset the phone.

- Complete the flashing and flexing process.

- Flashing procedure only

The phone can be upgraded by standalone EMMI box with flash PROM inserted. The EMMI box no need to connect to the PC or upload the flexing file.

- Connect v3688+ radio to the EMMI box through Buttplug cable.

- Insert test SIM card and power on the phone, enter test mode by press “#” key for 3 seconds and then press flash command “7536778#” to flash the phone.

- The phone will be power off after flashing completed.

- Power on the phone, enter test mode by press “” key for 3 seconds then press “19#” to check the s/w version is upgraded.

- The important things before Flashing and after Flashing

- Please notice that the battery capacity is not in low battery status (low battery) during flashing and flexing process. Otherwise, the process may be interrupted by low voltage power down. And the flash EPROM of phone may be damaged.

- All memory (phone book, user feature settings and timer) stored in the phone will be erased after reset the phone by test mode command “57#”.

- Please transfer the phone book memory (by transfer card) before flashing or reset the phone.

- The phone may show “Phone Failed See Supplier” after flashing. You need to flexing the phone with a correct flex ID (F1, F2, F3, F4 and F5” for v3688+).

- The phone must be flexing after flashing process.

- Equipment and cables

One set of software upgrade equipment as list below:

- PC with OS (MS DOS or Win 3.1 or Win 95).

- A dial up program (PROCOM PLUS version 2.01is recommended)

- EMMI box SLN3577D f/w version: 70.48.34

- V3688+ s/w version: CO.06.37 in one EMMI box (provided by Motorola).

- GSM data cable used to connect the PC to the EMMI box. (DB 9 female to 25 male cable. The following is the connections for the cable:

| - EMMI end 25 pin male DSUB | - PC end (to COM 1) 9 pin female DSUB |
| --- | --- |
| - 2 | - 3 |
| - 3 | - 2 |
| - 7 | - 5 |

- SPN 4221B power adapter for EMMI box.

- SKN 4858A buttplug cable.

- A flexing file v3688cke.fdf. (provided by Motorola)